# UASat Parts List

Science Team

f/7.0 (45 cm focal length) cassegrain telescope with a 15 cm diameter primary mirror primary

external baffling 2 cm from the edge of the primary mirror

hyperbolic secondary mirror is mounted at the 1/3 point of the primary, supported by curved diagonal vanes

fold mirror behind the primary mirror

Power and Data cables

# Science Instrument Box

Photometry Experiment, a spring-loaded 50/50 beam splitter will be pushed into the light path to divert all incoming light through a focal reducing lens and filter wheel

filter wheel will be rotated by a computer-controlled stepper motor and will contain the standard Johnson UBVRI filter set

0.1 mm silicon photodiode

a spring will pull the photometry beam splitter back into its default position

cylinder lens to squeeze image sideways into a line which falls on the photodiode array

dichroic beam splitter directly in front of the CCD

On-Board Computer (OBC)

CCD controller

## T.T.C. Team

4 ??? antennae, thin wires mounted at 45 from aperture-end

??? tape-measure-like antenna, ~ 60 cm long extended from launch interface-end

Power and Data cables

## G.N.C. Team

4 Reaction Wheels, 15 cm diameter x 5 cm thick

Torque Rods

Magnetometer

5 Sun Sensors: ~ 3 cm square thin patches, one placed on aperture end, one on shuttle-interface end, and three on the outer solar structure 120 deg. apart (every fourth body panel)

Horizon Sensors

Momentum Wheel(s)

Power and Data cables

## P.G.D. Team

Solar Panels

Batteries contained in pressure vessel

Voltage/Current Regulator

Power and Data cables

## D.C.H. Team

Mother-board

Power and Data cables

## S.T.I. Team

Some components contained in the Science team’s instrument box

Power and Data cables

## M.S.A. Team

Inner Telescope Tube,    ~46 cm long, 19cm inner diameter, 21 to 23 cm outer diameter

12-sided aluminum I-beam outer truss, ~46 cm long, ~47 cm between opposing corners

12 aluminum outer-body panels, ~ 0.2 cm thick

12-sided aperture-end plate, ~ 0.2 cm thick

12-sided launch vehicle interface-end plate, ~ 0.8 cm thick

12-sided shelf plate, ~ 0.5 cm thick, placed midway between aperture and interface plates

Aluminum Marmon plate, ~ 9 cm diameter, ~ 5 cm thick

Temperature transducers, Analog Devices: AD590

Cabling to provide 4 to 30 Volts and receive ~ 300mAmps to/from each temperature transducer.